SprinKlr-8

ESP32 8-Zone Sprinkler Controller

Design & Manufacturing Specification  |  v1.0  |  April 2026

| Target Board | SprinKlr-8 — Custom ESP32-WROOM-32E, 8-ch SPST relay, USB-C, SMD |
| --- | --- |
| MCU | Espressif ESP32-WROOM-32E-N4 (4MB Flash, 520KB SRAM, 2.4GHz WiFi + BT) |
| Relay Type | 8× SPST-NO SMD gull-wing, 5V coil, 5A @ 250VAC contacts (HF46F-G/5-HS1 / LCSC C165255) |
| Power Input | 5V via USB-C (standard phone charger) ¦ Relay contacts switch external 24VAC |
| Programming | USB-C — CH340C onboard UART bridge, no external adapter needed |
| Target Volume | 1,000 units ¦ JLCPCB PCBA + LCSC component sourcing |
| Est. Unit Cost | ~$7.10 fully assembled at 1,000 units (BOM + PCB + assembly) |
| PCB Size | ~90mm × 65mm (2-layer, ENIG or HASL finish) |

# 1.  Executive Summary

This document specifies the design and production plan for SprinKlr-8: a compact, low-cost ESP32-based irrigation controller with 8 independently switched zones. The board is optimized for volume production at JLCPCB using all-SMD components sourced through LCSC.

Key improvements over the reference design (ACEIRMC B0DTK2PB26):

- USB-C port integrated on-board — no external CP2102 programmer required

- SPST-NO relays replace SPDT — reduces per-relay cost and PCB footprint by ~20%

- Compact 90×65mm form factor vs. 131×82mm reference board

- Identical firmware pinout — existing sprinkler_controller.ino runs without modification

- 9× LED zone/power indicators for field diagnostics

- 5V USB-C powered — any phone charger drives the board in the field

# 2.  Board Design Overview

## 2.1  Power Architecture

The board is powered entirely from the USB-C port at 5V DC. This is a deliberate design choice:

- Eliminates need for onboard AC-DC converter — reducing cost, BOM complexity, and safety certification scope

- 24VAC from the irrigation transformer passes only through the relay contacts — no AC voltage is present on the PCB

- A standard 5V/2A USB-C phone charger provides ample current for the ESP32 (~240mA peak) and up to 8 relay coils (~70mA each = 560mA max all on simultaneously)

- USB-C also serves as the programming interface — the same cable powers and flashes the device

Note: If USB power is unavailable at the installation site, a 5V DC barrel jack footprint can be added at no BOM cost as an alternative power input. The firmware does not require any changes.

## 2.2  Relay Architecture

Each of the 8 zones is controlled by one SPST-NO (normally open) SMD relay. The choice of SPST over SPDT:

- Saves ~$0.05–0.08 per relay at volume — $0.40–0.64 per board saved

- Reduces PCB footprint — SPST relays have a smaller land pattern than SPDT equivalents

- Perfectly suited for sprinkler solenoid valves — valves only need a make/break contact, the NC terminal is unused in SPDT designs

The ULN2803A Darlington array drives all 8 relay coils from a single SO-18 IC. It includes integrated flyback diodes protecting the ESP32 GPIOs from relay coil back-EMF — no discrete diodes required.

## 2.3  USB-C & Programming

The CH340C USB-UART bridge connects the USB-C port to the ESP32's UART0 (TX/RX). This provides Arduino IDE-compatible flashing without any external hardware. Boot mode is entered by holding the onboard BOOT button while tapping RESET — identical to the reference board's IO0/EN button sequence.

Note: The ESP32-WROOM-32E is a pre-certified FCC/CE module (FCC ID: 2AC7Z-ESPWROOM32). Using this module means the final product may qualify for a simplified Supplier's Declaration of Conformity (SDoC) rather than full radio testing — a significant cost reduction for the commercial path.

## 2.4  Zone Indicators

Each zone has a dedicated 0402 green LED connected between 3.3V and the GPIO through a 470Ω current-limiting resistor. A red power LED indicates the board is powered. These allow quick visual confirmation of relay state without needing a phone or serial monitor.

## 2.5  GPIO Map (identical to firmware)

| Zone 1 | Zone 2 | Zone 3 | Zone 4 | Zone 5 | Zone 6 | Zones 7–8 |
| --- | --- | --- | --- | --- | --- | --- |
| GPIO 32 | GPIO 33 | GPIO 25 | GPIO 26 | GPIO 27 | GPIO 14 | GPIO 12, 13 |

Note: GPIO12 is an ESP32 strapping pin. The firmware initializes it LOW (relay OFF) before WiFi starts — this is already implemented in the existing sprinkler_controller.ino.

# 3.  Bill of Materials

All components are available through LCSC Electronics and can be ordered directly via the JLCPCB component library. Part numbers are current as of April 2026 — verify availability at time of order.

| Ref | Description | LCSC # | Qty | ×1 | ×100 | ×1,000 | Notes |
| --- | --- | --- | --- | --- | --- | --- | --- |
| U1 | ESP32-WROOM-32E-N4 (4MB Flash) | C701341 | 1 | $4.50 | $3.20 | $2.60 | Main MCU + WiFi/BT — FCC/CE pre-certified |
| U2 | CH340C USB-UART, SO-16 | C84681 | 1 | $0.55 | $0.28 | $0.20 | USB-C programming bridge |
| U3 | ULN2803ADWR 8-ch Darlington SO-18 | C5840 | 1 | $0.35 | $0.18 | $0.12 | Relay driver, includes flyback diodes |
| U4 | AMS1117-3.3 LDO SOT-223 | C6186 | 1 | $0.18 | $0.10 | $0.08 | 3.3V rail for ESP32 |
| K1–8 | SMD SPST-NO Relay 5V 5A (HF46F-G/5-HS1) | C165255 | 8 | $0.55 | $0.22 | $0.16 | 5A@250VAC, SMD gull-wing, G5NB footprint compatible |
| J1 | USB-C 16-pin SMD Receptacle | C165948 | 1 | $0.25 | $0.12 | $0.08 | Programming + 5V power input |
| J2–9 | KF301-2P 5.08mm Screw Terminal | C8465 | 8 | $0.22 | $0.12 | $0.08 | Zone output terminals (8× zone) |
| J10 | KF301-3P 5.08mm Screw Terminal | C8466 | 1 | $0.28 | $0.15 | $0.10 | COM + spare (24VAC common) |
| L1 | 100µH SMD Inductor (if DC reg used) | C408444 | 1 | $0.28 | $0.14 | $0.10 | Only needed if onboard 5V buck added |
| C1 | 100µF 16V Electrolytic SMD | C176672 | 1 | $0.18 | $0.08 | $0.06 | USB input bulk cap |
| C2–5 | 100nF 0402 MLCC ×4 | C1525 | 4 | $0.02 | $0.01 | $0.008 | Decoupling |
| R1–8 | 1kΩ 0402 resistor ×8 | C11702 | 8 | $0.01 | $0.005 | $0.003 | Relay drive signal resistors |
| R9–16 | LED current resistors 470Ω ×8 | C23190 | 8 | $0.01 | $0.005 | $0.003 | Zone LED resistors |
| D1–8 | LED 0402 green ×8 | C72043 | 8 | $0.05 | $0.025 | $0.018 | Zone active indicators |
| D9 | LED 0402 red (power indicator) | C84256 | 1 | $0.05 | $0.025 | $0.018 | Power-on LED |
| SW1 | BOOT tactile switch SMD | C318884 | 1 | $0.08 | $0.04 | $0.03 | Enter flash mode |
| SW2 | EN/RESET tactile switch SMD | C318884 | 1 | $0.08 | $0.04 | $0.03 | Reset ESP32 |
| — | PCB (2-layer, ~90×65mm, ENIG) | JLCPCB | 1 | $2.00 | $0.80 | $0.45 | 2-layer, HASL or ENIG finish |
| — | SMT Assembly (all SMD parts) | JLCPCB | 1 | $8.00 | $3.50 | $2.00 | JLCPCB Economic SMT |
| TOTAL | All components + PCB + assembly |  |  | $18.50 | $9.80 | $7.10 | Prices are estimates — verify via JLCPCB BOM tool |

Cost notes at 1,000 units:

- PCB cost assumes 2-layer, 90×65mm, HASL finish, standard lead time. ENIG adds ~$0.15/board.

- Assembly cost based on JLCPCB Economic SMT, single-sided, ~65 SMT placements + 9 through-hole terminals hand-soldered.

- Screw terminals (J2–J10) are through-hole — JLCPCB charges extra for THT assembly (~$0.015/joint). Consider SMD terminal alternatives to reduce this.

- ESP32 module dominates BOM cost (~36%). No cheaper certified WiFi+BT option exists at this volume.

# 4.  Manufacturer Comparison — 1,000 Units

| Manufacturer | Cost | Quality | Lead Time | Per-Board | Notes |
| --- | --- | --- | --- | --- | --- |
| JLCPCB + LCSC | ★★★★★ | ★★★★☆ | 7–15 days | $7–9 | Best all-in-one: PCB + assembly + LCSC sourcing in one order. Recommended. |
| PCBWay | ★★★★☆ | ★★★★★ | 10–20 days | $9–12 | Higher quality control, better for complex boards. Good for low-defect production. |
| NextPCB | ★★★☆☆ | ★★★★☆ | 10–18 days | $8–11 | Competitive pricing, less component sourcing integration than JLCPCB. |
| Seeed Fusion | ★★★★☆ | ★★★★☆ | 15–25 days | $9–13 | Good for IoT, has FCC/CE consultancy service. Useful for commercial path. |
| Local (US) | ★★★☆☆ | ★★★★★ | 5–10 days | $22–35 | MacroFab, Screaming Circuits. High cost but faster, no import risk. |

Recommendation: JLCPCB + LCSC

JLCPCB is the clear choice for this volume. The integrated workflow — design PCB in EasyEDA, source components from LCSC, order PCB + PCBA in one checkout — dramatically reduces project management overhead. At 1,000 units, JLCPCB's Economic SMT pricing makes it the lowest-cost option with acceptable quality for a non-safety-critical consumer product.

If quality control requirements increase (e.g., commercial sale with warranty obligations), consider moving to PCBWay or a Shenzhen direct contract manufacturer at 5,000+ units.

# 5.  Design → Manufacturing Workflow

## Step 1  —  Schematic in EasyEDA

- EasyEDA Standard (free, browser-based): https://easyeda.com

- Import LCSC part numbers directly — footprints and 3D models are auto-attached

- Key nets to implement: 5V_USB, 3V3, GND, RELAY_K1–K8, UART_TX, UART_RX, USB_D+, USB_D−

- Follow ESP32-WROOM-32E reference design for RF decoupling (39nF on antenna line, no copper pour under antenna area)

## Step 2  —  PCB Layout

- 2-layer board target: 90mm × 65mm

- Keep ESP32 module antenna clear of copper pour and relay fields

- Place relay coils away from USB-C connector (noise isolation)

- USB-C ESD protection: place PRTR5V0U2X or equivalent close to connector

- Relay terminal blocks on one edge for clean wiring

- Add board version, serial number silkscreen, and QR code linking to firmware repo

## Step 3  —  DFM Check

- Run JLCPCB DFM (Design for Manufacture) check before submitting

- Minimum trace: 0.127mm, min via: 0.3mm drill / 0.6mm pad — verify against design

- Check courtyard clearances around relay bodies — they are large SMD components

- Mark pin 1 indicators on all ICs in silkscreen

## Step 4  —  Generate Production Files

- Gerbers: use JLCPCB Gerber export plugin from EasyEDA

- BOM: export from EasyEDA with LCSC part numbers in 'Supplier Part' column

- CPL (Component Placement List): export XY coordinates + rotation from EasyEDA

- Cross-check BOM against LCSC inventory before ordering — mark any 'Extended' parts (extra $3/part type setup fee)

## Step 5  —  JLCPCB Order

- Upload Gerbers → configure: 2-layer, 90×65mm, HASL, 1.6mm, green, quantity 1,000

- Enable PCB Assembly → upload BOM and CPL files

- Review 3D preview in JLCPCB viewer before confirming

- Select 'Economic' assembly for lowest cost (vs 'Standard' which adds AOI and X-ray)

- Pay extra for AOI (Automated Optical Inspection) at $0.30/board — strongly recommended at 1,000 units

Note: JLCPCB ships assembled boards in anti-static trays. Request individual programming of each board if needed (JLCPCB offers this as an add-on for mass programming via JTAG/UART).

## Step 6  —  Incoming QC

- Flash 5–10 sample boards with relay_pin_finder.ino to verify all 8 relays operate

- Check USB-C programming works on sample boards before accepting the full order

- Measure current draw: should be ~80mA idle (ESP32 in WiFi mode), ~650mA with all 8 relays energized

# 6.  Commercial Path — FCC & CE Compliance

Because the ESP32-WROOM-32E module carries its own FCC Grant (FCC ID: 2AC7Z-ESPWROOM32) and CE marking, the compliance path for the final product is significantly simplified compared to using a bare chip.

## 6.1  US Market — FCC

| Requirement | Details |
| --- | --- |
| FCC Part 15B | Unintentional radiator (digital device) — self-declaration via SDoC. No lab testing required if module conditions are met. |
| Module conditions | Do not modify the ESP32 RF hardware or antenna. Do not add external antenna. PCB antenna clearance must be maintained as per module datasheet. |
| SDoC cost | ~$500–1,500 (legal/technical review, document preparation). No lab fees. One-time cost. |
| FCC label | Product must display FCC ID of the module: '2AC7Z-ESPWROOM32'. Add to PCB silkscreen and packaging. |

## 6.2  EU Market — CE

| Directive | Details |
| --- | --- |
| RED (Radio Equipment) | ESP32-WROOM-32E carries CE/RED marking. Self-declaration (DoC) is possible if module conditions met. |
| LVD (Low Voltage) | Board operates at 5V (USB-C) — below 75VDC threshold. LVD does not apply. |
| RoHS | JLCPCB PCBA is RoHS compliant by default. Confirm with supplier letter. |
| CE cost | ~$1,000–2,500 for DoC preparation and technical file. No mandatory lab testing if using certified module. |

Note: The 24VAC passing through the relay contacts does not change the board's regulatory status — it's an external circuit. The board itself operates at 5V SELV (Safety Extra Low Voltage).

## 6.3  Estimated Compliance Budget

| Item | Cost (one-time) | Notes |
| --- | --- | --- |
| FCC SDoC preparation | $500–1,500 | Attorney or compliance consultant |
| CE DoC preparation | $1,000–2,500 | Technical file + DoC document |
| Product liability insurance | $800–2,000/yr | Required for retail/Amazon sale |
| Total compliance (US + EU) | ~$2,300–6,000 | One-time. At 1,000 units = $2.30–6.00/board |

# 7.  Total Cost Summary — 1,000 Units

| Item | Total Cost | Per Unit |
| --- | --- | --- |
| PCB + PCBA + components (1,000 units) | ~$7,100 | $7.10 |
| FCC SDoC (estimated mid-range) | $1,000 | $1.00 amortized |
| CE DoC (estimated mid-range) | $1,500 | $1.50 amortized |
| 3D printed case (PLA, in-house) | ~$3,500 | $3.50 (material + time) |
| Packaging (plain white box) | ~$800 | $0.80 |
| Shipping from JLCPCB | ~$400 | $0.40 |
| TOTAL (with compliance + case) | ~$14,300 | ~$14.30/unit |

At a retail price of $49–69, the margin is healthy even at 1,000 units. Comparable commercial controllers (Rachio, RainBird WiFi) retail at $80–180 for 8 zones, providing strong competitive positioning.

# 8.  Recommended Next Steps

Immediate (prototype phase):

- Order 5 boards from JLCPCB to validate design before committing to 1,000 units (adds ~$2–3/board but saves scrapping a bad 1,000-unit run)Prototype:

- Create schematic and PCB layout in EasyEDA (free). Assign all LCSC part numbers before starting PCB to ensure component availability.Design:

- Update sprinkler_controller.ino to add USB-C detection and auto-baud (minor changes — GPIO map is unchanged)Firmware:

Before 1,000-unit order:

- Flash all 5 prototypes, run 48-hour soak test with real 24VAC valves connectedValidate:

- Submit Gerbers to JLCPCB DFM tool and resolve all warningsDFM:

- Verify all LCSC parts have sufficient inventory (>5,000 units) before orderingStock check:

Commercial path (if selling):

- Engage compliance consultant after prototype validation — $500–1,500, 2–4 weeksFCC SDoC:

- Consider Amazon FBA for initial sales — their FBA fees (~15% + fulfillment) are acceptable at this price pointRetail:

- At 5,000+ units, renegotiate directly with a Shenzhen contract manufacturer (Shenzhen electronics districts: Huaqiangbei) for 20–30% further cost reductionScale: